Small Projects Erosion and Sediment Control Plan for Earth Moving

When is an Erosion and Sediment (E&S) plan required?

Any earth disturbance is required to protect the site from accelerated erosion through the use of Best Management Practices (BMP’s). Some of these BMPs include, but not limited to filter fabric fence (silt fence), rock filters, stabilized entrances to site, grass buffers, diversion of upslope water, and seeding and mulching of disturbed areas.

Any disturbance over 5,000 square feet requires a written E&S plan to be developed and kept on site.

Additional permits may be required if the earth disturbance is over 1 acre (43,560 sq. ft.) over the life of the project.

An E&S Plan minimizes sediment runoff from an earthmoving activity. This in turn reduces sediment polluting nearby waterways, waterbodies, and wetlands.

The implementation and maintenance of erosion and sediment BMPs are required to minimize the potential of accelerated erosion and sedimentation.

What does and E&S Plan consist of?

E&S plans outline BMPs uses to minimize erosion problems associated with earthmoving activities.

A complete plan includes:

- Topographical Location Map

- Project Sketch Plan

- Details for E&S Controls

- Soils Map (https://websoilsurvey.sc.egov.usda.gov/App/WebSoilSurvey.aspx)

- Narrative description of the project

Please Print: Complete all spaces in the application. If there are any questions, please contact the conservation district for assistance.

Landowner:___________________________________________________________________

Mailing Address:_______________________________________________________________

City/State/Zip Code:____________________________________________________________

Project Name:_________________________________________________________________

Project Acres:__________________________ Disturbed Acres:__________________________

Project Location:_______________________________________________________________

Muncipality:___________________________________________________________________

Receiving Steam (http://www.depgis.state.pa.us/eMapPA/): _____________________________

Chapter 93 Designation:________________________

Is the earth disturbance within a floodway or 50 feet of a steam or waterway?

____YES	___NO

Project Description______________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Specific direction to project site:___________________________________________________

_____________________________________________________________________________

_____________________________________________________________________________

Estimated Start Date:_____________________ Completion Date:______________________________

Total Amount of Disturbed Area:

|  | Total Length | X | Total Width | X | = Area (sq. ft.) |
| --- | --- | --- | --- | --- | --- |
| Access Roads/ Driveways |  | X |  | X |  |
| Foundation/ Building |  | X |  | X |  |
| Lawn/ Landscaping |  | X |  | X |  |
| Other: |  | X |  | X |  |
| Total Area = sq. ft. |  |  |  |  |  |
| Total Area sq. ft. /43,560 = Disturbed Acres |  |  |  |  |  |

Soil Types: List the type(s) of soils found on the property and include a soils map (soils maps can be found on https://websoilsurvey.sc.egov.usda.gov/App/WebSoilSurvey.aspx):

____________________________________________________________________________

____________________________________________________________________________

Soil Limitations and how they will be addressed:

____________________________________________________________________________

____________________________________________________________________________

Construction Sequence:

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Temporary Best Management Practices (BMPs): This section details what temporary BMPs will be utilized during the project. Check each control that will be used.

______ Rock Construction Entrance			______ Temporary Seeding

______ Filter Fabric Fence (Silt Fence)		______ Mulching

______ Straw Bale Barrier 				______ Channel Lining (netting)

______ Rock Filter					______ Diverting Upslope Water

______ Rock Outlet Protection			______ Other

______ Sediment Trap				______ Other

CHECK:

______ All items checked above will be to DEP specifications.

______ Alternative controls will be maintained per manufacture’s specifications and are  attached.

Permanent Best Management Practices:

Prior to the completion of the project, state law requires that completion of any stage or phase of earth disturbance activity requires immediate seeding, mulching, or other protection from accelerated erosion and sedimentation. Implementations and maintenance of BMP’s (Best Management Practices) are required until the completion of permanent stabilization of the disturbed area. Types of permanent stabilization include: (1) uniform 70% perennial vegetative cover, with density capable of resisting erosion or (2) other acceptable BMPs that permanently minimize accelerated erosion and sedimentation.

All disturbed areas must be protected to prevent accelerated erosion. Soil cannot be left exposed. Revegetating an area should include the seeding mixture that will be used. Please provide how the site will be stabilized (i.e. vegetation, stone, pavement, ect).

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Maintenance Program:

All erosion control practices require maintenance to function properly. Please note the following maintenance procedures and check other applicable for control measures you will be using.

Until the site is stabilized, all erosion controls must remain in place and be maintained properly. Maintenance must include weekly inspections and inspections after each runoff event. All preventative and remedial maintenance work, including clean out, repair, and replacement must be done immediately.

After final stabilization has been achieved temporary erosion and sedimentation controls may be removed. Any disturbance created by the removal of these controls shall be stabilized. Stabilization is a uniform 70% vegetative cover or other type of cover that prevents accelerated erosion from the site (i.e. stone, pavement, ect).

Pollution Causing Geologic/Soil Conditions:

____YES	___NO

Thermal Impacts:

____YES	___NO